# EOSAT-1 LEOP & Commissioning Procedures

Launch and Early Operations Phase

# 1. Document Overview

This document provides comprehensive procedures for commissioning the EOSAT-1 spacecraft from separation through achievement of nominal operations. It follows the ECSS-E-ST-70-41C Packet Utilization Standard (PUS) for command structure and telemetry organization.

Scope: This manual covers:

- Pre-separation system verification and ground station readiness

- Separation sequence automation (Phase 0→1→2)

- Initial power-on and bootloader operations (Phase 3)

- Launch and Early Operations (LEOP) Phase 4: subsystem commissioning

- Commissioning Phase 5: full system checkout and payload activation

- Transition Phase 6: nominal operations readiness

Key Standards & References:

- ECSS-E-ST-70-41C: PUS standard (command/telemetry structure)

- EOSAT-1 Flight Software ICD v2.3

- EOSAT-1 Ground Segment Operations Manual

# 2. Pre-Separation Checks

Timeline: Perform all checks during the 24-hour window preceding launch. Repeat final verification items 2 hours before T-0.

## 2.1 Launch Vehicle Interface Verification

- Confirm separation mechanism arm pin removed and accounted for

- Verify launch vehicle umbilical interfaces locked and torque-checked

- Validate TTC monopole antenna connector impedance match (50Ω ± 5%)

## 2.2 Battery and Power State

Target state of charge (SoC) for separation: 95% ± 2%

- Confirm battery temperature 15°C–25°C (within acceptable charging window)

- Verify no load on unswitchable bus; all switchable lines OFF before umbilical disconnect

- Check battery management firmware version matches flight baseline

## 2.3 Separation Timer Configuration

The separation timer (param 0x0128) is pre-loaded with a 30-minute countdown. This timer:

- Starts automatically upon detection of separation event (MEMS accelerometer trigger)

- Cannot be stopped or reset by commands (hardware-locked)

- Triggers automatic enable of OBC and RX power at T+30 min (Phase 2)

Verify timer countdown is functional by requesting param 0x0128 from pre-separation checkout system.

## 2.4 Ground Station Readiness

- Confirm all ground tracking stations in acquisition-of-signal (AOS) standby

- Verify Mission Control System (MCS) receiving TTC beacon at nominal power level

- Load all command sequences and FDIR rules into MCS database

- Confirm RF link budget model updated for post-deployment antenna configuration

# 3. Separation Sequence (Phase 0→1→2)

This phase is fully automated. All systems remain powered down except for:

- MEMS accelerometer (detects launch vehicle separation)

- Separation timer hardware (independent countdown)

## 3.1 Separation Event (Phase 0→1)

### Sequence of Events

Trigger: Launch vehicle primary separation occurs ~300 s after launch. Secondary separation from kick stage occurs ~T+3000 s. The MEMS accelerometer detects the deceleration and triggers timer start.

- Timer value (param **0x0128**) begins counting down from 1800 (30 minutes)

- All subsystem power lines remain OFF

- TTC transponder remains in dormant beacon mode (passive RF reflection from monopole)

Ground Verification:

- Monitor param **0x0128** for countdown (should decrement every second)

- Confirm event **0x0001** (SEPARATION_DETECTED) logged in housekeeping

- Verify param **0x0127** = 1 (Phase 1 active flag)

CONTINGENCY 3.1-C1: Timer Fails to Start If param **0x0128** remains at 1800 or does not decrement for >60 s after separation, the hardware timer has failed. **Action:** Manually override to Phase 2 via command **S17.128** (FDIR_OVERRIDE_PHASE, arg=2). This forces immediate enable of OBC and RX power lines. Monitor **0x0128** continues to decrement in software emulation mode (less reliable; watch for power-on anomalies).

## 3.2 Initial Power-On (Phase 1→2)

Duration: Approximately 30 minutes after separation (when param 0x0128 reaches zero).

Automatic Actions:

- Hardware circuit automatically enables unswitchable 28V bus (OBC + RX lines)

- Battery output connects to bus; inrush current limited by soft-start circuitry

- OBC begins boot sequence from pre-programmed ROM bootloader

- RX transponder powers on in passive receiver mode (can accept commands)

Ground Verification (2–5 minutes after timer expiry):

- Monitor param **0x0105** (bus voltage) → should rise to 27.5–28.5V over ~10 s

- Wait for event **0x0050** (BUS_POWERED_UP) in housekeeping stream

- Confirm event **0x0051** (OBC_BOOTLOADER_ACTIVE)

- Verify param **0x0311** = 0 (OBC in bootloader, not application)

CONTINGENCY 3.2-C1: Bus Voltage Low (<26.5V) Bus voltage failure indicates insufficient battery charge or power distribution fault. **Immediate Action:** Check param **0x0101** (battery SoC). If SoC < 75%, verify solar array illumination: request param **0x0107** (solar array power). If in eclipse, wait for next sun exposure. If SoC > 75% and voltage still low, suspect battery connector fault or internal shorts. **Manual intervention required:** Contact launch operator to verify physical connections.

CONTINGENCY 3.2-C2: No OBC Boot Event (0x0051) If event **0x0051** is not received within 5 minutes of bus power-up, the OBC may be hung in early ROM code or failed to boot. **Action:** (1) Send command **S8.55** (OBC_BOOT_APP) with soft reset flag to trigger application entry. (2) If still no response, send **S8.53** (OBC_SWITCH_UNIT) to switch to redundant OBC-B. (3) Verify new unit boot sequence with event **0x0051**.

## 3.3 Bootloader Operations (Phase 3)

Duration: 1–10 minutes. The OBC is in bootloader state, accepting only a limited command set.

Active Telemetry:

- Service ID (SID) 10 (generic health housekeeping): bus voltage, battery SoC, temperatures

- Service ID (SID) 11 (OBDH diagnostics): memory checksums, watchdog status

- TTC subsystem active in beacon/low-rate mode (125 bps uplink, no downlink data)

Ground Procedure:

- Monitor HK from SID 10 and SID 11 to confirm power stability

- Once param **0x0105** stable for 30 s and no error events logged, proceed to application boot

- Send command **S8.55** (OBC_BOOT_APP, service 8 function_id 55) with parameter boot_flags=0x00

Expected response:

- Event **0x0052** (OBC_BOOTING_APPLICATION) within 5 s

- Param **0x0311** transitions from 0 → 1 (application now running)

CONTINGENCY 3.3-C1: Boot Failure (Event 0x0305) If event **0x0305** (OBC_BOOT_FAILED) is logged, the application image in flash is corrupted. **Action:** (1) Stay in bootloader mode. (2) Send **S8.55** OBC_BOOT_APP retry 3 times with 10 s delays. (3) If persistent, send **S8.53** (OBC_SWITCH_UNIT) to switch to OBC-B redundant unit. (4) Once OBC-B boots successfully, send **S19.4** (EVENT_ACTION_ADD) to create rule for automatic redundancy on OBC-A reboot failure.

# 4. LEOP Phase (Phase 4) — Launch & Early Operations

Duration: 1–7 days (typical mission timeline 3 days to nominal ops). During this phase, all subsystems are commissioned sequentially with continuous ground monitoring and automatic safeguards (FDIR) enabled.

## 4.1 EPS Commissioning

Objective: Verify battery charging logic, solar array power generation, and power distribution functionality.

### Initial Verification

- Request param **0x0101** (battery SoC) and confirm rising trajectory (should gain ~2–5% per orbital day in sun)

- Monitor param **0x0105** (bus voltage) remains 27.5–28.5V (stable ±0.5V)

- Monitor param **0x0106** (battery current) → should show charging when in sun, zero or negative in eclipse

- Request param **0x0107** (solar array power) → expect 0W in eclipse, 200–300W in full sun

### Power Line Testing

Test switchable power lines one at a time to verify isolation and fault protection:

- Send **S8.19** (EPS_POWER_ON, line=0 [ACS], power=1) → Verify param **0x011A** (ACS power status) = ON

- Request param **0x0118** (ACS line current) → expect 50–200mA (magnetorquers + sensors energized)

- Send **S8.20** (EPS_POWER_OFF, line=0) → Confirm **0x011A** = OFF and **0x0118** drops to zero

- Repeat for lines 1 (TCS), 2 (TTC), 3 (Payload), checking corresponding current parameters **0x0119**, **0x011B**, **0x011C**

### Charge Rate Configuration

Set battery charge controller to auto mode:

- Send **S8.23** (EPS_SET_CHARGE_RATE, rate_mode=0) for automatic temperature-compensated charging

- Verify param **0x0110** (charge controller state) = AUTO

CONTINGENCY 4.1-C1: Low SoC Declining If SoC is below 80% and declining (negative trend), the battery is not charging despite sun exposure. **Possible causes:** (1) Solar array not deployed or partially blocked. (2) High eclipse duration (high orbital inclination). **Action:** Request eclipse duration forecast from flight dynamics. If eclipse < 40 min, SoC should recover naturally. If eclipse > 40 min or SoC declining below 60%, reduce all non-essential loads (payload OFF, TTC to low rate). Monitor for next sunlit pass.

CONTINGENCY 4.1-C2: Overcurrent on Power Line If event **0x0109** (OVERCURRENT_DETECTED) is logged with line parameter, that line has a short circuit. **Action:** (1) Send **S8.21** (EPS_RESET_OVERCURRENT, line=X) to clear latch. (2) Send **S8.20** (EPS_POWER_OFF, line=X) to isolate the line. (3) Check FDIR rule for auto-shedding; if not active, manually manage payload/ACS power to prevent cascade failures. (4) Attempt power re-enable after 60 s; if short repeats, leave line OFF and route to redundant line if available.

CONTINGENCY 4.1-C3: Bus Voltage < 26.5V Under Load Sustained low voltage indicates insufficient battery capacity or excessive current demand. **Action:** (1) Send **S8.25** (EPS_LOAD_SHED, stage=1) to reduce switchable lines (payload, ACS to min state). (2) Verify bus voltage recovers to >27V. (3) Check eclipse duration; if eclipse upcoming, reduce loads pre-emptively. (4) If voltage recovers, gradually restore loads one at a time and monitor. If voltage remains low, escalate to stage 2 shedding (**S8.25**, stage=2) which disables all non-critical subsystems.

## 4.2 TTC (Telemetry & Telecommand) Commissioning

Objective: Verify RF link integrity, antenna deployment, and establish high-rate data downlink.

### Ground Contact & Antenna Deployment

The TTC subsystem is powered from the unswitchable bus and boots automatically. It initially operates in beacon mode (monopole antenna, low power).

- Wait for first ground station pass; confirm ground receives beacon signal

- Once RF link confirmed, send **S8.69** (TTC_DEPLOY_ANTENNA) command to extend helix antenna

WARNING: Antenna deployment is ONE-TIME IRREVERSIBLE. Verify all ground stations ready to receive before executing.

- Monitor for event **0x050D** (ANTENNA_DEPLOYED) and param **0x0520** = 1

- Confirm link margin param **0x0503** > 3 dB (indicates good RF path)

### High Data Rate Activation

Once antenna confirmed deployed:

- Send **S8.65** (TTC_SET_DATA_RATE, rate=1) to switch from 125 bps to 64 kbps downlink

- Verify param **0x0502** (TX data rate) = 64 kbps

- Confirm carrier lock and bit sync from ground RF receiver

CONTINGENCY 4.2-C1: Antenna Deploy Failure If event **0x050D** is not received within 5 minutes of **S8.69** command, the antenna release mechanism failed. **Action:** (1) Remain in beacon mode. (2) Operate at low data rate (125 bps) indefinitely. (3) Send **S8.69** retry up to 3 times with 10 s delays. (4) If still no deploy event, mark antenna as failed and transition to long-commissioning timeline with reduced downlink capacity. (5) Payload imaging will be limited; only priority high-value scenes downlinked.

CONTINGENCY 4.2-C2: Poor Link Margin (<3 dB) Low link margin indicates weak RF path (poor antenna pointing or solar eclipse affecting solar panels = less TX power). **Action:** (1) Send **S8.68** (TTC_SET_TX_POWER, power=HIGH) to increase transmitter output. (2) Wait for next station pass and remeasure **0x0503**. (3) If margin improves >3 dB, confirm high power active. (4) If margin still poor, verify antenna pointing (AOCS attitude) and check for ground receiver issues (request site RF verification).

CONTINGENCY 4.2-C3: No Carrier Lock / Sync Failure If ground receiver cannot achieve bit synchronization, the TTC transponder may be in wrong mode or transponder hardware failed. **Action:** (1) Send **S8.64** (TTC_SWITCH_TRANSPONDER, unit=B) to switch to redundant receiver unit. (2) Wait for next pass and confirm lock on unit B. (3) If locked successfully, keep unit B active and mark unit A as failed. (4) If both units fail to lock, suspect modulation/frequency issue; check OBC oscillator trim params and request telemetry from OBC clock subsystem.

## 4.3 OBDH Health Check

Objective: Verify OBC firmware stability, memory integrity, and watchdog operation.

- Request param **0x0300** (OBC operational mode) → confirm = 0 (NOMINAL)

- Request param **0x0302** (CPU load) → should be 20–40% (task scheduling normal)

- Request param **0x0303** (memory utilization) → should be < 30%

- Send **S8.59** (OBC_ENABLE_WATCHDOG) to arm watchdog timer (60 s timeout)

- Send **S8.61** (OBC_RUN_DIAGNOSTIC) and await event **0x030B** (DIAGNOSTIC_COMPLETE) with pass/fail status

CONTINGENCY 4.3-C1: High CPU Load (>60%) Elevated CPU load suggests task scheduling anomaly or infinite loop in firmware. **Action:** (1) Send **S8.51** (OBC_MEMORY_SCRUB) to clear any bit-flip corruption (SEU from radiation). (2) Wait 5 min and re-request **0x0302**. (3) If load decreases, SEU was likely cause; continue monitoring. (4) If load remains high, send **S17.0** (OBC_SOFT_RESET) to reboot OBC. (5) If load still high after reboot, suspect firmware bug; escalate to flight software team.

CONTINGENCY 4.3-C2: High Reboot Count If param **0x030A** (OBC reboot count) > 3 within the last 24 h, the OBC is unstable. **Action:** (1) Check event log for **0x0305** (BOOT_FAILED) or **0x03XX** error codes. (2) If reboots correlate with high CPU load, suspect firmware infinite loop; perform memory scrub (**S8.51**). (3) If reboots are random, suspect watchdog timeouts; increase watchdog timeout via **S8.60** (OBC_SET_WATCHDOG_TIMEOUT, timeout_seconds=120). (4) If reboots persist, switch to OBC-B via **S8.53** (OBC_SWITCH_UNIT) and place OBC-A in safe state.

## 4.4 AOCS Commissioning

Objective: Progress from safe detumble through coarse sun acquisition to nominal nadir pointing. This is the most complex subsystem with multiple mode transitions.

### SAFE_BOOT → DETUMBLE

Initial State: AOCS is powered but in standby after OBC boot.

- Send **S8.0** (AOCS_SET_MODE, mode=1 [SAFE_BOOT]) to activate safe attitude control

- AOCS automatically transitions 1 → 2 (DETUMBLE) when it detects angular rate increase

- During DETUMBLE, reaction wheel control law uses sun vectors and magnetic field to damp rotation

- Monitor angular rates (params **0x0204**, **0x0205**, **0x0206**) for decreasing trend over 30–120 minutes

- Expected trajectory: rate from ~1 deg/s → 0.1 deg/s over first orbit

### DETUMBLE → COARSE_SUN

Once rates < 0.2 deg/s:

- AOCS automatically transitions to mode 3 (COARSE_SUN)

- Coarse sun sensor (CSS) provides crude sun vector; wheel momentum control maintains orientation

- Monitor param **0x0210** (AOCS mode) to confirm transition = 3

### Star Tracker Commissioning

Once in COARSE_SUN (mode 3):

- Send **S8.4** (AOCS_POWER_STAR_TRACKER, unit=1, power=ON) to enable star tracker #1

- Allow 60 seconds for boot and initial alignment

- Monitor param **0x0240** (ST1 state) for transition to 2 (TRACKING)

- Verify param **0x0241** (ST1 num_stars) > 4 (star tracker locked on constellation)

- Confirm param **0x0242** (ST1 residual error) < 1 arcminute

### COARSE_SUN to NOMINAL

Once ST1 tracking confirmed:

- Send **S8.0** (AOCS_SET_MODE, mode=4 [NOMINAL]) to transition to fine nadir pointing

- AOCS uses star tracker quaternion + reaction wheels for 0.1° attitude accuracy

- Verify mode transition (param **0x0210** = 4) within 10 seconds

- Confirm attitude error param **0x0217** < 1° (and trending to <0.1° within 1 minute)

### Momentum Management

Reaction wheels accumulate momentum from external torques (solar pressure, gravity gradient). Check momentum:

- Request param **0x0227** (total momentum magnitude)

- Normal range: 0–25 Nms (< 50% of max saturation of 50 Nms)

- If momentum approaching 40 Nms, send **S8.1** (AOCS_DESATURATE_WHEELS) to unload momentum via magnetic torquers

- Desaturation typically completes in 5–15 minutes; monitor momentum decreasing

CONTINGENCY 4.4-C1: Angular Rates Not Decreasing If param **0x0204/0x0205/0x0206** remain > 0.5 deg/s for > 2 hours in DETUMBLE mode, the control law is not damping rotation. **Possible causes:** (1) Magnetorquers disabled. (2) Reaction wheels not responding. (3) Attitude sensors providing bad data. **Action:** (1) Send **S8.9** (AOCS_ENABLE_MAGNETORQUERS) to ensure mag torquer coils are powered. (2) Verify param **0x0215** (MT enable flag) = 1. (3) Request reaction wheel speeds (params **0x0220**–**0x0223**) to confirm non-zero and changing. (4) If wheels not spinning, check ACS power line via **S8.19** (EPS_POWER_ON, line=0). (5) If rate damping still fails, switch to manual detumble (operator-commanded momentum dumping via **S8.1**).

CONTINGENCY 4.4-C2: Star Tracker Blind If param **0x0240** remains 1 (SEARCHING) or shows 0x02 (BLINDED) after 3 minutes, the star tracker cannot see enough stars. **Possible causes:** (1) Spacecraft is sun-facing (sun exclusion zone). (2) Star tracker optics contaminated. (3) Spacecraft attitude too far from star field. **Action:** (1) Wait for next orbital pass into night side (sun exclusion zone ends every ~45 min). (2) If ST1 still blind on night side, power ST1 OFF (**S8.4**, unit=1, power=OFF) and power ST2 ON (**S8.5**). (3) Monitor ST2 state (param **0x0250**) for TRACKING. (4) If both trackers blind, operate in COARSE_SUN mode indefinitely and defer fine pointing commissioning.

CONTINGENCY 4.4-C3: High Momentum (>45 Nms) Accumulated wheel momentum indicates saturating angular momentum that will cause loss of attitude control if not shed. **Action:** (1) Send **S8.1** (AOCS_DESATURATE_WHEELS) immediately. (2) Monitor momentum param **0x0227** decreasing over next 5–20 minutes. (3) If momentum continues rising, suspect persistent external torque (e.g., solar pressure asymmetry). (4) Check attitude error **0x0217** for drift; if drifting significantly, may indicate sun-pointing error. (5) If desaturation takes > 30 minutes, increase magnetorquer strength via **S8.2** (AOCS_SET_MAGNETORQUER_STRENGTH, strength=HIGH).

CONTINGENCY 4.4-C4: Cannot Reach Nominal Mode If AOCS refuses to transition to mode 4 (NOMINAL) or transitions but attitude error **0x0217** > 2°, the fine pointing control law is failing. **Possible causes:** (1) Star tracker not tracking (verify **0x0240** = 2 and **0x0241** > 4). (2) Reaction wheels saturated or unresponsive. (3) Attitude propagation diverging. **Action:** (1) Remain in COARSE_SUN mode and continue payload operations at reduced precision (1–2° pointing error acceptable for wide swath imaging). (2) Request reaction wheel speed telemetry to confirm wheels responsive to commands. (3) Send **S8.1** (desaturate) and re-attempt mode transition after momentum < 20 Nms. (4) As fallback, implement coarse-pointing ground operations where imaging is scheduled for predicted sun-favorable geometry.

## 4.5 TCS (Thermal Control System) Commissioning

Objective: Verify thermal sensor network and heater controller functionality.

### Thermal Zone Monitoring

Monitor all 10 temperature zones (params 0x0400–0x0409):

- Battery zone (**0x0400**) → target 10–30°C

- OBC zone (**0x0401**) → target 15–35°C (acceptable during sun, may cool during eclipse)

- Other zones (0x0402–0x0409): payload cooler, reaction wheels, transmitter, solar panels → verify trends reasonable (±5°C per orbit max)

### Heater Activation

TCS heaters maintain minimum temperatures during eclipse:

- Send **S8.40** (TCS_ENABLE_HEATER, circuit=0 [battery]) to turn ON battery heater

- Send **S8.44** (TCS_SET_HEATER_SETPOINT, circuit=0, on_threshold=10°C, off_threshold=15°C) to configure thermostat

- Verify param **0x040E** (battery heater duty cycle) cycles 0–100% as zone temperature oscillates around setpoint

- Repeat for OBC heater (circuit=1): **S8.40**, then **S8.44** with setpoints on=5°C, off=10°C

CONTINGENCY 4.5-C1: Zone Overtemp (>40°C) If any zone exceeds 40°C, heater may be stuck ON or spacecraft in excessive sun soak. **Action:** (1) Send **S8.42** (TCS_DISABLE_HEATER, circuit=X) for the affected heater to turn OFF. (2) Check spacecraft attitude (param **0x0217**, **0x0218**) for large sun-angle error. (3) If attitude error large, send **S8.0** (AOCS_SET_MODE) to attempt attitude correction. (4) Monitor temperature trend downward. (5) If temperature continues rising despite heater OFF, may indicate radiator blocked; escalate to flight dynamics to plan sun-favorable attitude.

CONTINGENCY 4.5-C2: Zone Undertemp (<5°C) If zone temperature drops below 5°C despite heater active, heater output insufficient or zone sensor misaligned. **Action:** (1) Verify heater enable flag: request param **0x040A** (heater 0 status) = 1 (ON). (2) If heater already ON, increase heater power or lower setpoint via **S8.44** (on_threshold=-5°C). (3) Monitor duty cycle **0x040E** (should be cycling or steady high); if stuck at 0%, heater circuit may be open. (4) If sensor alignment issue suspected, cross-check with neighboring zone temp; if neighboring zone nominal, isolate this zone from critical operations.

CONTINGENCY 4.5-C3: Heater Not Responding If sending **S8.40** or **S8.44** produces no effect (heater status doesn't change), heater circuit controller may be unresponsive. **Action:** (1) Verify TCS power line ON: send **S8.19** (EPS_POWER_ON, line=1 [TCS]). (2) Request param **0x040A** (heater enable) to confirm command was received. (3) If param unchanged after command, suspect TCS subsystem communication failure. (4) Perform TCS reset: send **S17.0** (soft reset) to reboot entire spacecraft (will require re-initialization of all subsystems). (5) If still unresponsive, operate with passive thermal control (no active heaters) and schedule operations to avoid eclipse zones.

# 5. Commissioning Phase (Phase 5) — Full System Checkout

All major subsystems are now verified. Phase 5 focuses on payload integration and fine-tuning of sensor performance.

## 5.1 Payload Commissioning

Objective: Bring imaging payload from standby to operational mode and validate image quality.

### Power-Up & Cooler Activation

- Send **S8.19** (EPS_POWER_ON, line=3 [payload]) to energize payload power line

- Send **S8.26** (PAYLOAD_SET_MODE, mode=1 [STANDBY]) to activate cooler

- Cooler draws ~30W and should reduce focal plane assembly (FPA) temperature from ~25°C → -15°C over 60–120 seconds

- Monitor param **0x0601** (FPA temperature) for negative trend

### Calibration

Once FPA reaches -15°C:

- Send **S8.37** (PAYLOAD_START_CALIBRATION) to initiate dark frame + flat field sequence

- Calibration takes ~30 seconds (cover shutter closed, internal flat lamp on)

- Await event **0x060A** (CALIBRATION_COMPLETE) in housekeeping

### Image Capture & Quality Assessment

Once calibration complete:

- Send **S8.26** (PAYLOAD_SET_MODE, mode=2 [IMAGING]) to open shutter and prepare for capture

- Send **S8.28** (PAYLOAD_CAPTURE_IMAGE, scene_id=1, lines=5000) to capture a test image

- Image stored in onboard storage; param **0x0605** (image_counter) increments

- Request per-band signal-to-noise ratio (SNR) params **0x0620**–**0x0623** (4 spectral bands)

- Expected SNR: > 30 dB in sun-lit scene (indicates good sensor health)

CONTINGENCY 5.1-C1: FPA Not Cooling If param **0x0601** (FPA temp) remains above -5°C after 2 minutes, cooler is not functioning. **Possible causes:** (1) Payload power line OFF. (2) Cooler compressor failed. (3) Thermal conduction path blocked. **Action:** (1) Verify payload power via param **0x011C** (payload current) should show ~30W draw. (2) Check param **0x0610** (cooler status) = 1 (RUNNING). If status = 0, cooler not started; retry **S8.26** (mode=1). (3) If param **0x0610** = 1 but temp not decreasing, suspect mechanical failure. (4) Continue operations with warm FPA (SNR will be degraded but imaging possible). (5) Thermal engineering analysis required to assess operational constraints.

CONTINGENCY 5.1-C2: Calibration Failure If event **0x060A** (CALIBRATION_COMPLETE) is not received within 60 seconds, calibration sequence hung. **Possible causes:** (1) Shutter or lamp driver failed. (2) FPA not ready. **Action:** (1) Send **S8.38** (PAYLOAD_STOP_CALIBRATION) to abort sequence. (2) Check param **0x0611** (FPA ready flag) = 1. (3) If FPA ready, retry **S8.37**. (4) If calibration fails again, operate without calibration (images will have residual dark current bias; SNR reduced by ~3 dB). (5) Request backup calibration at ground station post-commissioning.

CONTINGENCY 5.1-C3: Low SNR (<25 dB) Low SNR indicates weak signal or high noise in imaging sensor. **Possible causes:** (1) Scene too dark (spacecraft attitude pointing wrong direction). (2) Sensor gain set too low. (3) Detector quantum efficiency degraded. **Action:** (1) Verify spacecraft attitude **0x0217** < 1° (good nadir pointing). (2) Send **S8.35** (PAYLOAD_SET_GAIN, gain=HIGH) to increase signal amplification. (3) Recapture image with **S8.28** and check SNR. (4) If SNR still low, check solar illumination angle (low sun angle reduces scene brightness). (5) As workaround, extend integration time via **S8.36** (PAYLOAD_SET_INTEGRATION_TIME, time=HIGH). (6) If SNR cannot exceed 25 dB, sensor may be degraded from launch vibration; document and proceed with reduced sensitivity.

CONTINGENCY 5.1-C4: Image Data Corrupt If downlinked image fails CRC or shows random pixel patterns, data was corrupted during transfer. **Possible causes:** (1) Memory error (cosmic ray bit flip). (2) Downlink bit error rate (BER) too high. (3) Payload processor checksum failure. **Action:** (1) Check param **0x0625** (payload memory error count) for elevated values. (2) Send **S8.31** (PAYLOAD_MARK_BAD_SEGMENT, segment_id=X) to mark corrupted image as bad and prevent re-downlink. (3) Verify TTC link margin **0x0503** > 3 dB. (4) If link margin low, increase TX power via **S8.68** (TTC_SET_TX_POWER, power=HIGH). (5) Attempt new image capture and downlink. (6) If corruption persists, may indicate memory controller issue; run diagnostics **S8.61** and escalate.

## 5.2 Fine Pointing Verification

Objective: Validate AOCS slew capability and fine pointing accuracy with payload engaged.

### Fine Pointing Mode Setup

Send S8.0 (AOCS_SET_MODE, mode=5 [FINE_POINT]) to enable high-bandwidth attitude control:

- This mode combines star tracker + reaction wheels for 0.05° RMS attitude error

- Verify transition (param **0x0210** = 5) within 5 seconds

- Confirm param **0x0217** (attitude error) < 0.1°

### Slew Maneuver Test

Once in FINE_POINT mode:

- Send **S8.10** (AOCS_SLEW_TO_QUATERNION, qx=0.0, qy=0.0, qz=0.707, qw=0.707, rate=1.0 dps) to command 90° roll maneuver

- Monitor slew progress via params **0x0280**–**0x028D** (slew quaternion target, current, error)

- Expected duration: ~90 seconds for 90° maneuver at 1 dps

- Verify slew completion: param **0x0285** (slew state) = 3 (COMPLETE)

CONTINGENCY 5.2-C1: Cannot Achieve Fine Pointing If mode transition to 5 fails or attitude error remains > 1°, fine pointing not available. **Possible causes:** (1) Star tracker not tracking. (2) Reaction wheels saturated. (3) AOCS FW bug. **Action:** (1) Verify ST1 state **0x0240** = 2 (TRACKING). (2) Check momentum **0x0227** < 30 Nms. (3) If momentum high, send **S8.1** (desaturate) and retry mode 5. (4) If ST1 not tracking, revert to mode 4 (NOMINAL) with coarse pointing. (5) Accept 1° pointing error; payload imaging still viable with ground post-processing.

CONTINGENCY 5.2-C2: Slew Failure If slew does not complete (param **0x0285** remains 1 [IN_PROGRESS] for > 2 min), slew control failed. **Action:** (1) Check momentum **0x0227**; if > 45 Nms, wheels saturated. Send **S8.1** (desaturate) and retry slew. (2) If momentum OK, suspect reaction wheel response lag or AOCS FW glitch. (3) Send **S17.0** (soft reset) and re-enter FINE_POINT mode. (4) Re-attempt slew with smaller angle (10° instead of 90°). (5) If slew still fails, mark fine pointing as unavailable and operate in NOMINAL mode only.

## 5.3 Data Downlink Verification

Objective: Test high-rate image downlink and verify data integrity at ground station.

### Image Downlink Queue

Once image captured and verified at spacecraft:

- Send **S8.29** (PAYLOAD_QUEUE_DOWNLINK, scene_id=1) to queue test image for transmission

- TTC begins streaming image data via service 13 (large data transfer)

- At high data rate (64 kbps), image (~50 MB) transfers in ~7 minutes

- Monitor transfer progress in ground station; confirm image received with correct CRC

### Telemetry Dump Verification

Test low-rate housekeeping dump:

- Ground station executes service 15.9 (TM_DUMP) to request all housekeeping stores

- Verify receipt of all HK records (SID 10, 11, 12, etc.) with timestamps and no gaps

CONTINGENCY 5.3-C1: Download Stalls If image downlink stops mid-transfer, TTC link lost or data buffer overflowed. **Action:** (1) Verify ground station still receiving telemetry (HK stream should continue). (2) Check link margin **0x0503**; if < 2 dB, increase TX power via **S8.68**. (3) Retry image downlink by re-sending **S8.29**. (4) If stall repeats, reduce data rate to 32 kbps via **S8.65** (rate=0) and retry. (5) If 32 kbps still stalls, suspect ground receiver issue; request RF site verification.

CONTINGENCY 5.3-C2: Data Corrupt After Download If received image fails CRC or displays artifacts, bit errors occurred in transmission. **Action:** (1) Check bit error rate (BER) telemetry from TTC subsystem. (2) If BER > 1e-6, increase TX power via **S8.68** (power=HIGH). (3) Reduce data rate to 32 kbps and retry image downlink. (4) If corruption persists, may indicate transponder RF power stage issue. (5) Send **S8.64** (TTC_SWITCH_TRANSPONDER, unit=B) to switch to redundant unit and retry. (6) Archive corrupted image; continue commissioning with new images.

## 5.4 FDIR Verification

Objective: Confirm fault detection, isolation, and recovery rules are armed and effective.

### Fault Detection System Status

EOSAT-1 implements two FDIR layers:

- **S12:** Real-time parameter monitoring (threshold-based triggers)

- **S19:** Event-action rules (logical responses to anomalies)

Verify both layers active:

- Request param **0x0700** (S12 monitoring enabled) = 1

- Request param **0x0710** (S19 event-action enabled) = 1

### Load Shedding Configuration

Verify load shedding thresholds are set:

- Stage 1 threshold (low battery SoC): param **0x0720** = 30% (shed payload, reduce TTC rate)

- Stage 2 threshold (critical SoC): param **0x0721** = 20% (shed ACS, disable all non-OBC/RX loads)

### Connectivity Test

Perform end-to-end system health check:

- Send **S17.1** (PING command) from ground station

- OBC echoes PING response immediately

- If PING times out (no response in 10 s), ground-spacecraft link degraded

# 6. Transition to Nominal Operations (Phase 6)

Entry Criteria:

- All subsystems commissioned and verified stable for ≥24 hours

- Payload image quality validated (SNR > 30 dB, image transfer reliable)

- AOCS in NOMINAL or FINE_POINT mode with attitude error < 1°

- Battery SoC stable and rising (> 85%)

- All FDIR rules armed and tested

Transition Activities:

- Enable science data collection: payload transitions to regular imaging mode on scheduled basis

- Transition housekeeping reporting to nominal rates (once per orbit instead of continuous)

- Hand over operations from Launch Team (LEOP) to Science Operations Team

- Archive all commissioning data and generate mission-critical parameter baseline

# Appendix A: Command Quick Reference

The following table lists all Service 8 (device commands) function IDs used in commissioning procedures:

| Function ID | Service | Command Name | Description |
| --- | --- | --- | --- |
| 0 | S8 | AOCS_SET_MODE | Set AOCS control mode (SAFE_BOOT/DETUMBLE/COARSE_SUN/NOMINAL/FINE_POINT) |
| 1 | S8 | AOCS_DESATURATE_WHEELS | Unload reaction wheel momentum via magnetorquers |
| 4 | S8 | AOCS_POWER_ST1 | Power star tracker #1 ON/OFF |
| 9 | S8 | AOCS_ENABLE_MAGNETORQUERS | Enable magnetic torquer control |
| 10 | S8 | AOCS_SLEW_QUATERNION | Command attitude slew to target quaternion |
| 19 | S8 | EPS_POWER_ON | Enable switchable power line |
| 20 | S8 | EPS_POWER_OFF | Disable switchable power line |
| 23 | S8 | EPS_SET_CHARGE_RATE | Configure battery charge controller mode |
| 26 | S8 | PAYLOAD_SET_MODE | Set payload to STANDBY/IMAGING/SAFE |
| 28 | S8 | PAYLOAD_CAPTURE_IMAGE | Capture image from focal plane array |
| 40 | S8 | TCS_ENABLE_HEATER | Enable thermal heater circuit |
| 53 | S8 | OBC_SWITCH_UNIT | Switch to redundant OBC |
| 55 | S8 | OBC_BOOT_APP | Transition from bootloader to application |
| 64 | S8 | TTC_SWITCH_TRANSPONDER | Switch to redundant TTC transponder unit |
| 69 | S8 | TTC_DEPLOY_ANTENNA | Deploy helix antenna (IRREVERSIBLE) |

# Appendix B: Telemetry Verification Checklist

Key telemetry parameters to monitor during commissioning. All parameter IDs are in hex format (0xXXXX):

| Param ID | Parameter Name | Expected Nominal Value | Verification Point |
| --- | --- | --- | --- |
| 0x0101 | Battery SoC (%) | 95% @ sep → rising | Phase 2: 24-hour trend |
| 0x0105 | Bus Voltage (V) | 27.5–28.5 | All phases: continuous |
| 0x0107 | Solar Array Power (W) | 200–300 sun / 0 eclipse | Phase 2: per-orbit cycle |
| 0x0128 | Separation Timer (s) | 1800 → 0 | Phase 0: countdown |
| 0x0210 | AOCS Mode | 4 (NOMINAL) | Phase 4: after ST1 lock |
| 0x0217 | Attitude Error (deg) | < 0.1° NOMINAL / < 1° COARSE | Phase 4–6: continuous |
| 0x0240 | ST1 State | 2 (TRACKING) | Phase 4: after boot |
| 0x0300 | OBC Operational Mode | 0 (NOMINAL) | Phase 3+: after boot |
| 0x0311 | OBC App Running Flag | 1 (APP running) | Phase 3: after boot cmd |
| 0x0503 | TTC Link Margin (dB) | > 3 dB | Phase 4: per pass |
| 0x0601 | Payload FPA Temp (°C) | -15 cooled / > -5 safe | Phase 5: cooler test |
| 0x0620-0x0623 | Payload Band SNR (dB) | > 30 dB each band | Phase 5: image quality |

# Appendix C: Event Reference

Key telemetry events generated during normal commissioning sequence. All event IDs are in hex format (0xXXXX):

| Event ID | Event Name | Meaning | Expected When |
| --- | --- | --- | --- |
| 0x0001 | SEPARATION_DETECTED | MEMS accel triggered; timer started | Phase 0→1 |
| 0x0050 | BUS_POWERED_UP | Unswitchable bus enabled, voltage rising | Phase 1→2 |
| 0x0051 | OBC_BOOTLOADER_ACTIVE | OBC in ROM bootloader, ready for boot cmd | Phase 2→3 |
| 0x0052 | OBC_BOOTING_APPLICATION | OBC executing boot_app command | Phase 3, after S8.55 |
| 0x050D | ANTENNA_DEPLOYED | TTC antenna helix extended, ready for comms | Phase 4, after S8.69 |
| 0x060A | PAYLOAD_CALIBRATION_COMPLETE | Payload cal dark frame + flat field done | Phase 5, after S8.37 |

# Document Control

Document Title: EOSAT-1 LEOP & Commissioning Procedures

Document ID: EOS-LOP-001

Revision: A

Date: 2026-04-05

Classification: CONFIDENTIAL

Prepared by: EOSAT Mission Operations Team

Approved by: EOSAT Project Manager