Debris-Impact Training Scenario — Critique & Design Review

Subject: eps_debris_impact.yaml (HERMES-3 / SA-2B, 22-min cascade)

Prepared 2026-06-04

# Executive summary

The scenario is a well-written training narrative — narrative voice, command sequencing, FD-authorisation discipline, and the explicit scoring rubric are all professional. As an artefact that should actually drive the SMO simulator, however, it has two distinct categories of issue: mechanical compatibility (subsystem names, failure types, parameter identifiers and the scoring/contact-window schema are not what the engine reads, so most of the YAML is silently ignored), and operational realism (the documented behaviour of real LEO debris strikes — most directly Sentinel-1A in 2016 — is much milder and faster-recovering than the 22-minute multi-subsystem cascade depicted, and real onboard FDIR safes the spacecraft in seconds-to-minutes for the bus conditions described, removing the operator from the loop). The corrected YAML configs/eosat1/scenarios/debris_impact_hermes3.yaml addresses the mechanical issues. The design issues are largely intentional dramatisation for training, but should be flagged in the briefing so trainees don't draw incorrect lessons about how a real strike presents.

# 1. Line-by-line content critique

Walking through the YAML against the SMO ScenarioEngine schema and subsystem models. Severity tags: [Breaking] = the event will silently no-op when the scenario runs; [Schema] = the field is parsed but not read by the engine; [Realism] = mechanically OK but contradicts real-world reference behaviour; [Style] = nit.

## Top-level fields

- Line 1 name: "HERMES-3 Debris Impact — Alpha-7" — fine; the ScenarioDefinition reads this. [OK]

- Line 4 contact_window_s: 660 — [Schema] ScenarioDefinition has no contact_window_s field; this is read and discarded. If you want contact-window-based scoring, the scenario engine needs the field added.

- Lines 6–13 briefing — [OK] solid context-setting; classification banner is good practice for training material.

## F-1 — EPS solar-array cell loss (T+00:00)

- Line 22 subsystem: eps — [OK] the SMO EPS model is named eps.

- Line 23 failure: solar_array_cell_loss — [Breaking] the EPS model's inject_failure has no solar_array_cell_loss branch. Remap to sa_b_fail (the SMO has independent sa_a_fail/sa_b_fail failures) or extend the model. Currently this silently does nothing.

- Line 26 target: SA-2B — [Schema] passed as a kwarg to inject_failure; ignored unless the model defines a panel-level handler. The SMO models the EPS arrays as A/B only, so the per-panel level isn't there to be commanded.

- Lines 29–55 four action: set_parameter events — [Breaking] the scenario engine did not handle set_parameter at all (it silently fired with no effect). This is now fixed: it accepts a numeric param_id. Your symbolic names (sa_total_output_w, bus_voltage_v, battery_discharge_current_a, power_margin_w) do not resolve, because the scenario engine has no name→id registry. Use the hex IDs from configs/eosat1/parameters.yaml (bus voltage is 0x0107, battery SoC 0x0101). Note: sa_total_output_w and power_margin_w do not exist as SMO parameters at all — they would need to be defined and emitted by the EPS model first.

## F-2 — ADCS (T+02:15)

- Line 61 subsystem: adcs — [Breaking] the SMO uses aocs not adcs. The instructor-UI dropdown does this translation client-side, but scenarios do not — so the engine never finds a model and silently no-ops. Rename throughout.

- Line 62 failure: gimbal_drive_fault — [Breaking] not a handled AOCS failure. Closest analogue is rw_bearing / rw_seizure (and AOCS now logs a WARNING for unknown failure names, so this would at least show in the log). The HERMES-3 design appears to have steerable arrays with a SADA, which the SMO doesn't model.

- Lines 68–101 five set_parameter events using symbolic names (rw_b_speed_rpm, pointing_error_deg, disturbance_torque_nm, etc.) — [Breaking] none of these are parameter names the SMO emits. RW speeds are at 0x0207 – 0x020A, and after the recent remediation pass RW temperatures are now in SID 2 at 0x0218 – 0x021B.

## F-3 — TCS heater circuit (T+04:40)

- Line 108 failure: heater_circuit_cut — [Breaking] not a handled TCS failure. The SMO TCS has htr_battery_stuck_on, htr_battery_open_circuit, cooler_failed, and sensor drifts — pick one that matches the intent, or extend the model.

- Line 110 onset: ramp — [Breaking] FailureManager onset modes are step, gradual, and intermittent; ramp is silently rejected and the failure stays at zero magnitude. Use gradual.

- Lines 121–140 four set_parameter events — same Breaking issue as F-1/F-2: symbolic names. battery_temp_c maps to 0x0407. battery_charge_efficiency_pct and active_heater_circuits are not modelled.

## F-4 — COMMS / TTC (T+07:10)

- Line 146 subsystem: comms — [Breaking] the SMO uses ttc, not comms. Rename.

- Line 147 failure: tx_power_reduction — [Breaking] not a handled TTC failure. Closest are uplink_loss (loss-of-link symptom — now correctly suppresses the downlink after recent fixes), pa_thermal, or antenna_deploy_failed. A direct "reduce TX power" failure would need to be added to the TTC model.

- Lines 153–179 four symbolic set_parameter events — [Breaking] of these only an analogue of "downlink rate" is plausibly mappable; uplink_margin_db, command_accept_rate_pct are not modelled.

## F-5 — OBC protective shutdown (T+11:00)

- Line 185 subsystem: obc — [Breaking] the SMO uses obdh, not obc. Rename.

- Line 186 failure: pme_undervoltage_trip — [Breaking] not in the OBDH model. Closest semantically is bus_failure (CAN bus to subsystems fails) plus EPS-side safing which after recent fixes does cascade properly via safe_mode_eps load shedding. The corrected YAML uses bus_failure so the recovery (operator switches CAN bus via S8 func 54) is exercisable.

- Lines 191–225 five set_parameter events with string values (EMERGENCY, OFF, THRUSTER_COARSE) — [Breaking] [Realism] the corrected set_parameter handler now accepts string values for the param dict, but EOSAT-1 has no thrusters (per models/tcs_basic.py comments), so a "THRUSTER_COARSE" mode isn't reachable.

## F-6 — Safe-mode lockout (T+22:00)

- Line 232 failure: safe_mode_lockout — [Breaking] not a defined failure type. The SMO has watchdog_reset which produces a real OBC reboot after the recent watchdog fix and is the closest analogue.

- Line 235 condition: "battery_soc_pct < 60" — [Breaking] the condition evaluator splits on whitespace and matches the first token against the string form of a numeric param id, so symbolic names never resolve — the lockout event would never fire. Use 0x0101 < 60 instead.

## expected_responses block (lines 239–328)

- [Schema] the model defines expected_responses as a flat list of {category, description} dicts; your extra fields (step, fd_authorisation_required, commands, verification) are loaded onto each entry but ignored by the debrief logic. The narrative value to a human reading the YAML is real; the engine just doesn't act on it.

- [Schema] the commands lists are operator mnemonics (EPS_LOADSHED_TIER2, RW_DESATURATE B), not PUS service/subtype/func_id tuples. To exercise them through the procedure runner, map each to {service, subtype, data_hex} or use the activity-types catalogue.

## scoring block (lines 331–375)

- [Schema] the scoring block is loaded into the parsed dict but ScenarioDefinition does not retain it, and the debrief uses only response-counting heuristics. The criteria are excellent — they're worth porting into the scenario engine as a real scoring extension. Worth filing as a defect.

# 2. Design realism review

This section grades the scenario against published behaviour of real LEO MMOD events and current FDIR practice. The headline finding is that the scenario's dramatic 22-minute cascade across five subsystems is more severe than any documented MMOD event on a comparable spacecraft and removes the autonomous-safing behaviour that defines real operations. That is a legitimate training abstraction provided trainees understand it; flagging it explicitly in the briefing would be sound.

## 2.1  Plausibility of a single-impact 30 % cell loss

"37 of 120 cells" from one impact is a strong claim. The best-documented LEO debris hit is Sentinel-1A on 23 August 2016 17:07 UT: a few-mm impactor (estimated mass of a ~5 mm aluminium sphere) produced a ~40 cm visible damage zone and about 5 % of array power lost — roughly 280 W of the 5.9 kW array, permanently but with no FDIR safing triggered, and a star-tracker dropout of about five minutes before full attitude recovery. To get to 30 % cell loss in a single event you generally need the strike to initiate a sustained plasma arc between adjacent strings, which the array's own bias voltage then sustains and propagates well beyond the kinetic crater — a documented phenomenon in ground tests but only inferred (not confirmed) on-orbit. If you want the magnitude in the scenario to be physically credible, the briefing should name the sustained-arc mechanism rather than imply 37 cells were severed kinetically.

## 2.2  Bus voltage should be coupled, not commanded

The YAML forces bus_voltage_v = 23.1 the instant the cell loss is injected. On a real bus the regulator (PCDU + battery) holds voltage steady while the battery absorbs the load increase; voltage sags only when battery SoC falls under load. Even at Sentinel-1A's ~280 W loss there was no observable bus sag. A more faithful realisation would let the generation drop and let the SMO EPS battery + bus model derive the voltage trajectory; that is exactly what the existing EPS tick does without intervention.

## 2.3  Autonomous FDIR will safe the spacecraft long before T+22:00

Real spacecraft EPS FDIR uses N-consecutive-sample debouncing on undervoltage and trips within seconds-to-a-few-minutes; that's the entire point — round-trip ground response is too slow to protect the battery. After the recent SMO remediation pass, the simulator now actually does this: the FDIR rule eps.bus_voltage < 26 calls safe_mode_eps, which puts the EPS into safe mode and sheds non-essential loads. A 22-minute scenario with the operator in the loop is therefore realistic only if the scenario explicitly narrates that FDIR has been inhibited or has already tripped and recovered. Otherwise trainees will draw the incorrect lesson that there are 22 minutes for deliberation; on a real bus there are seconds.

## 2.4  Cascade ordering

Realistic order under partial-array loss is EPS (instantaneous on the array, buffered on the bus) → ADCS (asymmetric solar pressure / drag-torque growth over tens of seconds to minutes) → TCS (gradient growth over many minutes) → COMMS (only if explicitly load-shed). The YAML respects this ordering — EPS first, then AOCS, TCS, COMMS, OBDH — which is good. What is unrealistic is treating all five as roughly equal-weight failures occurring on minutes-scale intervals; the TCS effect is genuinely the slowest, and COMMS is normally unaffected. The OBDH "PME undervoltage trip" at T+11:00 is the kind of secondary effect that, in reality, should be the first autonomous response, not the fifth in the sequence.

## 2.5  Historical reference cases

Iridium-33 / Cosmos-2251 (10 Feb 2009) remains the canonical hypervelocity-collision reference — 11.6 km/s closing velocity, ~1,800 catalogued ≥ 10 cm fragments — but it is a destructive event, not a survivable partial-loss anomaly relevant to this scenario.

Hitomi / ASTRO-H (26 Mar 2016) is the canonical cascade-to-mission-loss case study, but the cascade was ground-induced — a spurious IRU rotation, mis-configured magnetic-torquer unload, and an uplinked thruster parameter that did not account for the deployed EOB mass properties accelerated rather than damped the spin until panels shed. The lesson is latent configuration error + ADCS cascade, not impact. A debris-impact scenario that culminates in mission loss should explicitly cite Hitomi as the mechanism, not bundle the dynamics as if they're a single physical chain.

ENVISAT (8 Apr 2012) abruptly went silent; ESA's investigation concluded the most likely cause was an internal power-regulator failure or short, possibly compounded by a failed safe-mode transition — but the root cause was never confirmed because telemetry was lost. This is a useful reference for irrecoverable EPS faults but not for cascade dynamics.

ISS Canadarm2 & Nauka radiator strikes demonstrate the typical end of the spectrum: a ~mm-class impactor punches a small hole and the system continues operating with a logged anomaly.

SOHO (25 Jun 1998) is the textbook ground-induced cascade — a missing command in an uplinked sequence left a gyro despun, then misdiagnosis produced a cascade to attitude / power / thermal loss. The HERMES-3 scenario's command-sequencing and FD-authorisation discipline is specifically designed to defend against this class of incident, which makes it a useful drill even if the framing is debris-impact rather than ground-error.

## 2.6  What the standards say

NASA-STD-8719.14C and IADC-02-01 Rev. 2 govern debris-mitigation rules (25-year orbital lifetime, end-of-mission passivation, conjunction-assessment); they do not prescribe scenario design. ECSS-Q-ST-30C Rev.1 is the European reference for FDIR design, failure-tolerance criticality, and what "safe mode" means — relevant when grading whether a scenario's FDIR timeline is consistent with an ECSS-compliant bus. NASA's flight-controller training programmes are explicit that scenarios must be physically faithful to the real vehicle's behaviour — they test operators' knowledge and identify weaknesses in procedures and flight rules, which is undermined when the scenario teaches behaviour the real vehicle does not exhibit.

## 2.7  Realism scorecard

Grading the scenario against seven concrete criteria distilled from the cited literature:

| Criterion | Verdict | Notes |
| --- | --- | --- |
| Magnitude of single-impact cell loss | Aggressive | 30 % loss from one strike is plausible only via sustained-arc propagation; Sentinel-1A precedent is ~5 %. |
| Bus voltage modelled vs forced | Forced | YAML hard-sets 23.1 V; a real bus sags only when battery SoC falls. Drop the set_parameter for bus_voltage and let the EPS model derive it. |
| Operator window vs FDIR speed | Unrealistic without narrative caveat | 22 min implies FDIR is absent. Real EPS undervoltage trips in seconds-to-minutes; brief trainees that FDIR is explicitly inhibited. |
| Cascade ordering | Good | EPS → ADCS → TCS → COMMS is the right physical order. OBDH trip is in the wrong slot — it would be earlier. |
| FD authorisation layering | Excellent | Three FD-authorised commands, marked discipline. Better than most published training material. |
| Telemetry signature fidelity | Mixed | Many forced values are made-up parameter names; star-tracker dropout pattern (5 min, gyro-only AOCS, ~0.7° peak yaw error) — the most distinctive Sentinel-1A signature — is not represented. |
| Recovery realism | Inflated | Historical events (Sentinel-1A, Canadarm2) recovered autonomously or with routine ground actions. Cascades to loss (Hitomi, SOHO) were ground-induced. Acknowledge in the briefing. |

# 3. Mapping table: what to change to make it run

The corrected version is at configs/eosat1/scenarios/debris_impact_hermes3.yaml — the table below summarises the substitutions made.

| Original | Replaced with | Why |
| --- | --- | --- |
| subsystem: adcs | subsystem: aocs | SMO model name |
| subsystem: comms | subsystem: ttc | SMO model name |
| subsystem: obc | subsystem: obdh | SMO model name |
| failure: solar_array_cell_loss | failure: sa_b_fail | Handled failure type |
| failure: gimbal_drive_fault | failure: rw_bearing | Closest handled analogue |
| failure: heater_circuit_cut | failure: cooler_failed | Closest handled analogue |
| failure: tx_power_reduction | failure: uplink_loss | Closest handled analogue |
| failure: pme_undervoltage_trip | failure: bus_failure | OBDH CAN-bus failure (recovery via S8 func 54) |
| failure: safe_mode_lockout | failure: watchdog_reset | Triggers a real OBC reboot |
| onset: ramp | onset: gradual | Valid onset name in FailureManager |
| parameter: sa_total_output_w | (not modelled) | Drop or extend EPS model + telemetry config |
| parameter: bus_voltage_v | param_id: 0x0107 | Recommended to drop and let model derive |
| parameter: rw_b_speed_rpm | param_id: 0x0208 | RW speed wheel index 1 |
| parameter: battery_temp_c | param_id: 0x0407 | temp_battery in SMO |
| condition: battery_soc_pct < 60 | condition: 0x0101 < 60 | Condition evaluator needs numeric ids |

# 4. Recommendations

- Adopt the corrected YAML debris_impact_hermes3.yaml for actual execution against the SMO. Keep the original alongside as a written design document.

- Annotate the briefing to acknowledge that the scenario compresses and dramatises what a real debris strike would look like, and cites Sentinel-1A as the realistic-magnitude reference point.

- File an SMO defect to extend ScenarioDefinition with the additional fields the YAML uses: contact_window_s, scoring, and the richer expected_responses schema (step, fd_authorisation_required, commands, verification). The criteria as written are unusually well-thought-out and worth preserving.

- For each unmapped failure name in the original (e.g. gimbal_drive_fault, heater_circuit_cut, tx_power_reduction, pme_undervoltage_trip), decide whether to (a) extend the model with that failure mode, or (b) accept the remapping. Both are reasonable; the choice depends on whether you want HERMES-3-like fidelity or just a working drill.

- Add the operator command mnemonics ("EPS_LOADSHED_TIER2", "RW_DESATURATE B", etc.) to configs/eosat1/activity_types.yaml with their real PUS service/subtype/func_id, so they can actually be executed through the MCS procedure runner.

- Consider sequencing the OBDH "undervoltage trip" earlier than T+11:00 to reflect real FDIR speed, or — if you want to retain the 22-minute window — narrate that the trip is explicitly inhibited.

# References

- Krag, H. et al. (2017). A 1 cm space debris impact onto the Sentinel-1A solar array. Acta Astronautica 137, 434–443.

- ESA / CLS MPC-S1 (2016). Sentinel-1A Debris Collision August 2016, MPC-0352 v1.0.

- ESA (2016). Copernicus Sentinel-1A satellite hit by space particle.

- NASA Orbital Debris Program Office (2010). The Collision of Iridium 33 and Cosmos 2251.

- JAXA (2016). Status of X-ray Astronomy Satellite Hitomi; Earth, Planets and Space 69:82 (2017).

- Fukushige, S. et al. (2007). Sustained arc from plasma excited by hypervelocity impact. Int. J. Impact Engineering.

- Wen, X. et al. (2018). Discharge effect of solar array under HVI. AIP Advances 8:105206.

- NASA HVIT (2019). Observations of MMOD Impact Damage to the ISS, NTRS 20190033989.

- ESA (2012). ESA declares end of mission for Envisat.

- SOHO Mission Interruption Joint ESA/NASA Investigation Board (1998). Final Report.

- NASA-STD-8719.14C. Process for Limiting Orbital Debris.

- IADC-02-01 Rev. 2 (March 2020). IADC Space Debris Mitigation Guidelines.

- ECSS-Q-ST-30C Rev.1 (2017). Space product assurance — Dependability.

- Aerospace Corp. TOR-2013-00295. Architectures for Lithium-Ion Based Power Subsystems.

- Tipaldi & Bruenjes (2017). Turning model-based FDIR theory into practice. EUCASS 2017.

- Huning, J. et al. (2011). A Review of Three Decades of Flight Controller Training. NTRS 20110014997.

Krag, H. et al., 'A 1 cm space debris impact onto the Sentinel-1A solar array', Acta Astronautica 137 (2017) 434–443.

ESA / CLS MPC-S1, 'Sentinel-1A Debris Collision August 2016,' technical note MPC-0352 v1.0, 18 Oct 2016.

NASA Orbital Debris Program Office, 'The Collision of Iridium 33 and Cosmos 2251: The Shape of Things to Come,' NTRS 20100002023.

JAXA, 'Status of X-ray Astronomy Satellite Hitomi (ASTRO-H),' 8 Apr 2016; 'An analysis of the 2016 Hitomi breakup event,' Earth, Planets and Space 69:82 (2017).

Fukushige, S. et al., 'Preliminary study on sustained arc due to plasma excited by hypervelocity impact of space debris on the solar array coupon,' Int. J. Impact Engineering (2007). Wen, X. et al., AIP Advances 8:105206 (2018).

IADC-02-01 Rev. 2, 'IADC Space Debris Mitigation Guidelines,' March 2020.

ECSS-Q-ST-30C Rev.1, 'Space product assurance — Dependability,' 15 Feb 2017.

NASA-STD-8719.14C, 'Process for Limiting Orbital Debris.'

NASA HVIT, 'Observations of MMOD Impact Damage to the ISS,' NTRS 20190033989; 'Canadarm2 micrometeoroid hole,' NASA / Universe Today, 31 May 2021.

Huning, J. et al., 'A Review of Three Decades of Flight Controller Training,' NTRS 20110014997.

Aerospace Corp., 'Architectures for Lithium-Ion Based Power Subsystems,' TOR-2013-00295. Tipaldi & Bruenjes, 'Turning model-based FDIR theory into practice,' EUCASS 2017.

SOHO Mission Interruption Joint ESA/NASA Investigation Board, Final Report, 31 Aug 1998.